SoNDe Detector: Fundamental Ideas and General Concept

Günter Kemmerling

Forschungszentrum Jülich GmbH, Jülich Centre for Neutron Science (JCNS) and Peter Grünberg Institute (PGI), Wilhelm-Johnen-Strasse, 52428 Jülich, Germany

Within the SoNDe project it is intended to develop a new position sensitive neutron detector. The development aims at instruments at future spallation sources which require medium large position sensitive detectors that particulary offer a high count rate capability. The small angle scattering instrument SKADI at the future ESS is such an instrument which expects a peak rate of scattered neutrons of up to 20 MHz on an area of a square meter. There, a position resolution of 6 mm and partly also 3 mm or even lower is required together with a time resolution in the µs regime and a high neutron detection efficiency of 80% or more.

The high rate requirement is very challenging and exceeds the capability of todays neutron detectors used for small angle scattering. Major limiting factors for the count rate capability of these detectors are the occupancy of the area of a detector unit with neutrons, the pulse processing electronics as well as the duration of the neutron detection process. Thus, for the SoNDe detector a concept has been chosen that is based on a fast neutron detection process and a higher segmentation of the detection area with individual and independent channels. A pixelated Li-glass scintillator should be used for the neutron capture. It has a fast decay time of the light emission and provides a high absorption efficiency for neutrons. The size of the scintillator pixels should match the size of the channels of multi anode photomultipliers (MaPMT), which will be used for the detection of the scintillator light. Individual and independent detection channels will be provided by an accurate positioning of the scintillator pixels on the MaPMT channels and building a light barrier between the scintillator pixels. As the light produced by the neutron capture with Li-glass scintillators and detected by photomultipliers deliver a range of pulse heights that can be assigned to neutrons, a simple and fast pulse processing electronics can be used to identify neutrons and discriminate against γ-background. There are two types of MaPMTs available from the company HAMAMATSU with channel sizes of 6 mm and 3 mm that can be applied to achieve the desired position resolution for the SKADI instrument. With the H8500 MaPMT and an ASIC based electronics from the company IDEAS a prototype detector has been build and used for first measurements to prove and optimize the SoNDe detector concept.

The SoNDe project is a collaboration between the Forschungszentrum Jülich (Germany), the Laboratoire Léon-Brillouin (France), the European Spallation source and the Universtity of Lund (Sweden) and Integrated Detector Electronics AS (IDEAS, Norway). We gratefully acknowledge finacial support by the European Union within the Horizon 2020 framework (project number 654124).

E-mail: G.Kemmerling@fz-juelich.de